DEPARTMENT OF COMPUTER & SOFTWARE ENGINEERING

COLLEGE OF E&ME, NUST, RAWALPINDI

Subject Name

Digital Signal Processing

Lab Number

1

SUBMITTED TO:

LE Sundas Ashraf

SUBMITTED BY:

Student Name

- Wahaaj Nasir

Reg#413238

DE- 44 Dept C&SE

Objectives:

Processing in MATLab

Related Topic/Chapter in theory class:

Basics Of Digital Signal Processing

Hardware/Software required:

Hardware: PC

Software Tool: MATLab

Task 1:

Record a voice with default sampling rate and perform following operations:

(a)

Solution:

%Task 1%

%Sampling frequency Fs is no of samples per second

recorder = audiorecorder(8000, 8, 1);

fs = 8000 %Adding fs as variable becoz mei har jagah use kar raha

disp("Start Recording");

recordblocking(recorder, 5);

disp("Stop Recording");

play(recorder)

myVoice = getaudiodata(recorder);

reverse_audio = flipud(myVoice);

pause(7)

sound(reverse_audio, 8000)

pause(7);

subplot(3, 1, 1)

plot(myVoice)

xlabel("Samples");

ylabel("Amplitude");

title("Original Sound")

subplot(3, 1, 2)

plot(reverse_audio)

xlabel("Samples");

ylabel("Amplitude");

title("Reverse Sound")

The audiorecorder records the voice and outputs it as a column matrix. This is done flipped using the ‘flipud’ command.

(b)

Add 2 cosines into the audio with frequency ranging from 1kHz to 1.5Hz, play plot the signal using subplot.

Solution

%Task2

f1 = 1000;

f2 = 1500;

%Determine length of the signal

N = length(myVoice);

t = (0:N-1)/fs; %Goes to 5 seconds

cos_sig_1 = cos(2 *pi * f1 *t)';

cos_sig_2 = cos(2 *pi *f2 *t)';

cos_added_voice = myVoice + cos_sig_1 + cos_sig_2;

sound(cos_added_voice, 8000)

plot(cos_added_voice)

xlabel("Samples");

ylabel("Amplitude");

title("Cos Added")

Due to the high frequency of the cosine function, the original sound is lost.

(c)

Divide the array Of your audio into two equal parts, save the divided arrays into two

variables (let say a and b) and add both (a+b), play the resultant voice and plot the signal

using subplot.

Solution

%Task 3

partA = myVoice(1:20000, 1);

partB = myVoice(20001:end, 1);

sound(partA, 8000)

pause(4)

sound(partB, 8000)

pause(4)

reconstruct = partA+partB;

sound(reconstruct, fs)

subplot(4, 1, 1)

plot(partA);

title("Part A")

xlabel("Samples")

ylabel("Amplitude")

subplot(4, 1, 2)

plot(partB);

title("Part B")

xlabel("Samples")

ylabel("Amplitude")

subplot(4, 1, 3)

plot(reconstruct);

title("Reconstruct")

xlabel("Samples")

ylabel("Amplitude")

subplot(4, 1, 4)

plot(myVoice);

title("Original Signal")

xlabel("Samples")

ylabel("Amplitude")

As you can see, we separate Part A and Part B from the original signal. Adding them increases the amplitude of the signals.

(d)

Perform [(a*2) + (b*0.5)] play the resultant voice and plot the signal using subplot.

Solution

%Task 4

partA_2 = partA .* 2;

partB_0_5 = partB .* 0.5;

recon = partA_2 + partB_0_5;

sound(recon, fs)

subplot(4, 1, 1)

plot(partA_2);

title("Part A_2")

xlabel("Samples")

ylabel("Amplitude")

subplot(4, 1, 2)

plot(partB_0_5);

title("Part B_0_5")

xlabel("Samples")

ylabel("Amplitude")

subplot(4, 1, 3)

plot(recon);

title("Reconstruct")

xlabel("Samples")

ylabel("Amplitude")

From the figure and sound byte, we see that A has a louder voice while the B part is quieter.

(e)

Add 4 cosines with frequency ranging from (5Hz to 4kHz) into audio signal, play the

resultant voice and plot the signal using subplot.

Solution

%Task 5

f1 = 5;

f2 = 200;

f3 = 1000;

f4 = 2500;

%Determine length of the signal

N = length(myVoice);

t = (0:N-1)/fs; %Goes to 5 seconds

cos_sig_1 = cos(2 *pi * f1 *t)';

cos_sig_2 = cos(2 *pi *f2 *t)';

cos_sig_3 = cos(2 *pi *f3 *t)';

cos_sig_4 = cos(2 *pi *f4 *t)';

new_sound = myVoice + cos_sig_1 + cos_sig_2 + cos_sig_3 + cos_sig_4;

subplot(2, 1, 1)

plot(myVoice)

title("Original Sound")

xlabel("Sample")

ylabel("Amplitude")

subplot(2, 1, 2)

plot(new_sound)

title("Cos Added Sound")

xlabel("Sample")

ylabel("Amplitude")

Using a lower frequency we can see the effect on the signal

(f)

Downsample the audio signal 5 times, dropping every other sample

Solution

%Task 6

downsampleVoice_1 = downsample(myVoice, 2); %Divide by 2

downsampleVoice_2 = downsample(downsampleVoice_1, 2); %Divide by 2

downsampleVoice_3 = downsample(downsampleVoice_2, 2); %Divide by 2

downsampleVoice_4 = downsample(downsampleVoice_3, 2); %Divide by 2

downsampleVoice_5 = downsample(downsampleVoice_4, 2); %Divide by 2

subplot(6, 1, 1)

plot(myVoice)

title("Original Sound")

xlabel("Sample")

ylabel("Amplitude")

subplot(6, 1, 2)

plot(downsampleVoice_1)

title("Downsample Once")

xlabel("Sample")

ylabel("Amplitude")

subplot(6, 1, 3)

plot(downsampleVoice_2)

title("Downsample Twice")

xlabel("Sample")

ylabel("Amplitude")

subplot(6, 1, 4)

plot(downsampleVoice_3)

title("Downsample Thrice")

xlabel("Sample")

ylabel("Amplitude")

subplot(6, 1, 5)

plot(downsampleVoice_4)

title("Downsample Four Times")

xlabel("Sample")

ylabel("Amplitude")

subplot(6, 1, 6)

plot(downsampleVoice_5)

title("Downsample Five Times")

xlabel("Sample")

ylabel("Amplitude")

The samples are reduced by a factor of 2 for 5 times

Task 2

Take two gray-scale images of same size and perform following operations:

(a)Read the images in two matrices.

(b)Take transpose Of the matrix and display the resultant images using subplot.

(c)Add two images and display the result.

(d)Read an image then multiply matrix of image with 0.5 and display the result.

Solution

%%Task 2 - Images

Cat = imread("Cat.jpg");

Cat1_Gray = rgb2gray(Cat);

Cat2 = imread("Cat 2.jpg");

Cat2_Gray = rgb2gray(Cat2);

subplot(2, 2, 1)

imshow(Cat)

title("Cat 1")

subplot(2, 2, 2)

imshow(Cat2)

title("Cat 2")

subplot(2, 2, 3)

imshow(Cat1_Gray)

title("Cat 1")

subplot(2, 2, 4)

imshow(Cat2_Gray)

title("Cat 2")

%%

Cat1_transpose = (Cat1_Gray)';

Cat2_transpose = (Cat2_Gray)';

Cat_Add = Cat1_Gray + Cat2_Gray;

Cat_1_Mult = Cat1_Gray .* 0.5;

subplot(2, 2, 1)

imshow(Cat1_transpose)

title("Cat 1 Transpose")

subplot(2, 2, 2)

imshow(Cat2_transpose)

title("Cat 2 Transpose")

subplot(2, 2, 3)

imshow(Cat_Add)

title("Added Two Images")

subplot(2, 2, 4)

imshow(Cat_1_Mult)

title("Cat 2")

Converting to Grayscale using rgb2gray

Task 3

Read an video and display it using imread

Solution

%Task 3

Video = VideoReader("Video.mp4")

numFrames = ceil(Video.Duration * Video.FrameRate);

frames=cell(numFrames, 1)

for k=1:numFrames

frame = read(Video, k);

frames{k} = frame;

imshow(frame)

end

Using a for loop configure for the framerate of the video we can display the video

Home Task

Reverse the frames and play the video in reverse

Solution

%%

rev_vid = frames(end:-1:1);

for k =1:numFrames

frame =rev_vid{k};

imshow(frame)

end

We use the frame array of cells and reverse the video